# CanOEs - CAN Communication GUI

# User Manual

Version: 1.0

Date: December 2025

Author: Dariusz Piskorowski

__________________________________________________

# Table of Contents

[Introduction](#1-introduction)

[Requirements](#2-requirements)

[Getting Started](#3-getting-started)

[Main Interface Overview](#4-main-interface-overview)

[Connection Panel](#5-connection-panel)

[Send Panel](#6-send-panel)

[Received Messages Panel](#7-received-messages-panel)

[Tabs Overview](#8-tabs-overview)

- [Main Tab](#81-main-tab)

- [Filters Tab](#82-filters-tab)

- [Periodic Tab](#83-periodic-tab)

- [Timing Tab](#84-timing-tab)

- [History Tab](#85-history-tab)

- [Predefined Tab](#86-predefined-tab)

- [Grouped Tab](#87-grouped-tab)

[Features](#9-features)

- [Theme Toggle](#91-theme-toggle)

- [Export Logs](#92-export-logs)

- [Message Coloring](#93-message-coloring)

- [ASCII Preview](#94-ascii-preview)

- [ID Comments](#95-id-comments)

[How-To Guides](#10-how-to-guides)

[Troubleshooting](#11-troubleshooting)

__________________________________________________

# 1. Introduction

CanOEs (CAN Open Environment Software) is a graphical user interface for CAN and CAN FD communication using Vector VN1640A hardware. It provides an intuitive way to send, receive, filter, and analyze CAN messages.

## Key Features:

CAN Classic (up to 8 bytes) and CAN FD (up to 64 bytes) support

Standard (11-bit) and Extended (29-bit) ID support

Message filtering by ID, range, or mask

Periodic message transmission

Message history and predefined messages

Dark/Light theme support

Export logs to TXT file

__________________________________________________

# 2. Requirements

## Hardware:

Vector VN1640A CAN interface (or compatible VN16xx device)

## Software:

Windows operating system

Vector XL Driver Library (vxlapi64.dll)

Python 3.8 or higher

Required Python packages:

- tkinter (usually included with Python)

__________________________________________________

# 3. Getting Started

**Connect Hardware**: Connect your Vector VN1640A to the computer via USB

**Install Drivers**: Ensure Vector drivers are installed

**Run Application**: Execute `python can_gui.py`

**Select Channel**: Choose the CAN channel (1-4)

**Select Mode**: Choose CAN or CAN FD mode

**Connect**: Click the "Connect" button

__________________________________________________

# 4. Main Interface Overview

The application window is divided into several sections:

┌─────────────────────────────────────────────────────────────┐

│  [Toggle Theme]                              Window Title   │

├─────────────────────────────────────────────────────────────┤

│  CONNECTION PANEL                                           │

│  [Channel] [Baudrate] [Mode] [Connect/Disconnect]          │

├─────────────────────────────────────────────────────────────┤

│  SEND PANEL                                                 │

│  [ID] [Extended] [FD] [BRS] [Data] [Pad 00] [Send]         │

├─────────────────────────────────────────────────────────────┤

│  TABS: [Main] [Filters] [Periodic] [Timing] [History]...   │

│  ┌─────────────────────────────────────────────────────┐   │

│  │                                                     │   │

│  │              TAB CONTENT AREA                       │   │

│  │                                                     │   │

│  └─────────────────────────────────────────────────────┘   │

├─────────────────────────────────────────────────────────────┤

│  STATUS BAR: TX: 0  RX: 0  Errors: 0  [Status]             │

└─────────────────────────────────────────────────────────────┘

__________________________________________________

# 5. Connection Panel

Located at the top of the window, this panel controls the connection to the CAN hardware.

## Controls:

| Control | Description |
| --- | --- |
| **Channel** | Dropdown to select CAN channel (1, 2, 3, or 4). Corresponds to physical channels on VN1640A. |
| **Baudrate** | CAN bus speed selection: 125k, 250k, 500k, 1M bit/s |
| **FD Baudrate** | Data phase baudrate for CAN FD: 1M, 2M, 4M, 5M, 8M bit/s |
| **CAN FD Checkbox** | Enable CAN FD mode (allows up to 64 bytes per message) |
| **Connect Button** | Establishes connection to selected channel. Changes to "Disconnect" when connected. |

## Connection Status:

**Disconnected**: Gray status, "Connect" button visible

**Connected**: Green status, "Disconnect" button visible

**Error**: Red status with error message

__________________________________________________

# 6. Send Panel

This panel allows you to compose and send CAN messages.

## Controls:

| Control | Description |
| --- | --- |
| **ID (hex)** | Message ID in hexadecimal (e.g., `123`, `7DF`, `18DA00F1`) |
| **Extended ID** | Checkbox - enable for 29-bit extended CAN ID |
| **FD** | Checkbox - send as CAN FD frame (allows more than 8 bytes) |
| **BRS** | Checkbox - Bit Rate Switch (faster data phase in CAN FD) |
| **Data (hex)** | Message payload in hexadecimal, space-separated (e.g., `01 02 03 04`) |
| **Byte Counter** | Shows current/maximum bytes (e.g., `8/8 bytes` or `8/64 bytes` for FD) |
| **Pad 00** | Button - fills remaining bytes with zeros (to 8 for CAN, 64 for CAN FD) |
| **Send** | Button - transmits the message |

## Data Length Validation:

**CAN Classic**: Maximum 8 bytes

**CAN FD**: Maximum 64 bytes

Counter turns **red** if data exceeds the limit

Message will not be sent if data is too long

## Examples:

Standard CAN message:

ID: `123`

Extended ID: ☐ (unchecked)

FD: ☐ (unchecked)

Data: `01 02 03 04 05 06 07 08`

CAN FD message with BRS:

ID: `456`

Extended ID: ☐ (unchecked)

FD: ☑ (checked)

BRS: ☑ (checked)

Data: `01 02 03` → Click "Pad 00" → `01 02 03 00 00 00 ... 00` (64 bytes)

Extended ID message:

ID: `18DA00F1`

Extended ID: ☑ (checked)

Data: `02 10 01`

__________________________________________________

# 7. Received Messages Panel

Located in the Main tab, this panel displays all received and transmitted CAN messages.

## Toolbar Buttons:

| Button | Description |
| --- | --- |
| **▶ Start / ⏹ Stop** | Start or stop receiving messages |
| **Clear** | Clear all messages from the list |
| **Export TXT** | Save message log to a text file |
| **Auto-scroll** | Checkbox - automatically scroll to newest message |
| **Show Time** | Checkbox - display timestamp column |
| **Show ASCII** | Checkbox - display ASCII representation of data |
| **Color Messages** | Checkbox - enable/disable color coding of messages (TX=green, RX=blue, DIAG=orange) |
| **Edit Comments** | Open dialog to add/edit comments for message IDs |

## Message List Columns:

| Column | Description |
| --- | --- |
| **Time** | Timestamp when message was sent/received |
| **Dir** | Direction: TX (transmitted) or RX (received) |
| **ID** | Message ID in hexadecimal |
| **DLC** | Data Length Code (number of data bytes) |
| **Data** | Message payload in hexadecimal |
| **ASCII** | ASCII representation of data (if enabled) |
| **Flags** | Message flags: EXT (extended), FD, BRS |
| **Comment** | User-defined comment for this ID |

## Message Colors:

| Color | Meaning |
| --- | --- |
| **Green** | TX - Transmitted messages |
| **Blue** | RX - Received messages |
| **Orange** | DIAG - Diagnostic messages (known diagnostic IDs) |
| **Red** | ERR - Error frames |

__________________________________________________

# 8. Tabs Overview

## 8.1 Main Tab

The primary workspace containing:

Received Messages panel with message list

Real-time message display

Message filtering based on active filters

## 8.2 Filters Tab

Create and manage message filters to show or hide specific CAN IDs.

### Filter Types:

| Type | Description | Example |
| --- | --- | --- |
| **Single** | Match exact ID | ID = `0x123` |
| **Range** | Match ID within range | From `0x100` to `0x1FF` |
| **Mask** | Match IDs using bit mask | Base `0x700`, Mask `0x7F0` matches `0x700-0x70F` |

### Filter Controls:

| Control | Description |
| --- | --- |
| **Filter Name** | User-friendly name for the filter |
| **Filter Type** | Single, Range, or Mask |
| **Accept/Reject** | Accept = show matching, Reject = hide matching |
| **Enabled** | Checkbox to enable/disable filter |
| **Add Filter** | Create new filter |
| **Remove Selected** | Delete selected filter |
| **Enable All / Disable All** | Batch enable/disable all filters |

## 8.3 Periodic Tab

Configure messages to be sent automatically at regular intervals.

### Periodic Message Settings:

| Setting | Description |
| --- | --- |
| **ID** | Message ID (hex) |
| **Data** | Message payload (hex) |
| **Interval** | Time between sends in milliseconds |
| **Count** | Number of times to send (0 = infinite) |
| **Extended/FD/BRS** | Message flags |

### Controls:

| Button | Description |
| --- | --- |
| **Add** | Add new periodic message |
| **Remove** | Remove selected message |
| **Start All** | Start all enabled periodic messages |
| **Stop All** | Stop all periodic messages |

## 8.4 Timing Tab

Configure timing parameters for message transmission.

### Settings:

| Setting | Description |
| --- | --- |
| **Min Frame Gap** | Minimum time (ms) between consecutive transmissions |
| **TX Timeout** | Timeout for transmission confirmation |

## 8.5 History Tab

View and resend previously transmitted messages.

### Features:

Automatically records all sent messages

Double-click or select + "Resend" to retransmit

"Load" button copies message back to Send panel for editing

"Clear History" removes all history entries

### Columns:

| Column | Description |
| --- | --- |
| **Time** | When message was originally sent |
| **ID** | Message ID |
| **Data** | Message payload |
| **Flags** | EXT, FD, BRS flags |

## 8.6 Predefined Tab

Store frequently used messages for quick access.

### Default Predefined Messages:

**Diag 0x744** - `02 10 01` (Diagnostic session)

**Diag 0x744 Extended** - `02 10 03` (Extended session)

**OBD Engine RPM** - `0x7DF`: `02 01 0C` (Request engine RPM)

**OBD Vehicle Speed** - `0x7DF`: `02 01 0D` (Request vehicle speed)

**OBD Coolant Temp** - `0x7DF`: `02 01 05` (Request coolant temperature)

**Tester Present** - `0x7DF`: `02 3E 00` (Keep session alive)

### Controls:

| Button | Description |
| --- | --- |
| **Send** | Transmit selected predefined message |
| **Add Current** | Save current Send panel content as predefined |
| **Remove** | Delete selected predefined message |

## 8.7 Grouped Tab

View message statistics grouped by CAN ID.

### Columns:

| Column | Description |
| --- | --- |
| **ID** | Unique message ID |
| **Count** | Number of times this ID was seen |
| **Last Data** | Most recent payload for this ID |
| **Last Time** | Timestamp of most recent message |
| **Comment** | User-defined comment |

### Controls:

| Button | Description |
| --- | --- |
| **Refresh** | Update grouped view with latest data |
| **Clear** | Reset all grouped statistics |

__________________________________________________

# 9. Features

## 9.1 Theme Toggle

Click "Toggle Theme" button (top-left) to switch between:

**Light Theme**: White background, dark text

**Dark Theme**: Dark background, light text (easier on eyes)

## 9.2 Export Logs

Click **"Export TXT"** in the toolbar

Choose save location and filename

File contains all messages in human-readable format:

CanOEs - Message Log

Exported: 2025-12-05 14:30:00

=====================================

12:00:01.123  TX  0x123  8  01 02 03 04 05 06 07 08  ........

12:00:01.456  RX  0x456  4  11 22 33 44              ."3D      Response

## 9.3 Message Coloring

Messages can be automatically color-coded based on their type. This feature is enabled by default but can be toggled on/off.

### How to enable/disable coloring:

In the toolbar above the message list, find **"Color Messages"** checkbox

☑ **Checked** = Colors are applied to messages

☐ **Unchecked** = All messages displayed in default color (black/white depending on theme)

### How it works (when enabled):

Colors are assigned automatically when messages appear in the list

No additional configuration needed - it just works!

Repeated messages without data changes will gradually fade (become grayed out)

Messages with changing data stay in full color

| Color | Type | When Applied |
| --- | --- | --- |
| 🟢 **Green** | TX | Any message you transmit |
| 🔵 **Blue** | RX | Any message received from the bus |
| 🟠 **Orange** | DIAG | Messages with diagnostic IDs (0x7DF, 0x7E0, 0x7E8, 0x744, 0x74C, 0x700-0x703) |
| 🔴 **Red** | ERR | Error frames |
| 🔘 **Gray** | STALE | Repeated messages with unchanged data (after 5+ repetitions) |

### Visual Priority:

Error frames are always red

Diagnostic IDs override TX/RX color (shown in orange)

Stale/repeated messages fade to gray regardless of type

### When to disable coloring:

When you need maximum readability in certain lighting conditions

When preparing screenshots for documentation

When you prefer a cleaner, monochrome look

## 9.4 ASCII Preview

Enable "Show ASCII" checkbox to see ASCII representation of data bytes:

Printable characters (32-126) shown as-is

Non-printable characters shown as `.`

Example:

Data: `48 45 4C 4C 4F 00 01 02`

ASCII: `HELLO...`

## 9.5 ID Comments

Add custom comments to CAN IDs for documentation:

Click **"Edit Comments"** button

In dialog:

- Enter ID (hex): 744

- Enter Comment: ECU Diagnostic Request

- Click "Add/Update"

Comments appear in the message list and grouped view

### Default Comments:

| ID | Comment |
| --- | --- |
| 0x7DF | OBD-II Broadcast |
| 0x7E0 | OBD-II ECU Request |
| 0x7E8 | OBD-II ECU Response |
| 0x744 | Diagnostic Request |
| 0x74C | Diagnostic Response |

__________________________________________________

# 10. How-To Guides

This section provides step-by-step instructions for common tasks.

## 10.1 How to Connect to CAN Bus

Goal: Establish connection to your CAN network

Steps:

Plug Vector VN1640A into USB port

Wait for Windows to recognize the device

Launch CanOEs application

In **Connection Panel** (top of window):

- Select Channel: Usually 1 (check your physical connection)

- Select Baudrate: Must match your CAN network (common: 500k)

- Check CAN FD if your network uses CAN FD

- If CAN FD, select FD Baudrate (common: 2M)

Click **Connect** button

Status bar should show "Connected" in green

Troubleshooting:

If "Channel not found" → check USB connection

If no messages appear → verify baudrate matches other devices

__________________________________________________

## 10.2 How to Send a CAN Message

Goal: Transmit a single CAN message

Steps:

Ensure you are connected (see 10.1)

In **Send Panel**:

- Enter ID in hex (e.g., 123 or 7DF)

- Check Extended ID if using 29-bit ID

- Check FD for CAN FD message (more than 8 bytes)

- Check BRS for faster data rate (only with FD)

- Enter Data in hex with spaces (e.g., 01 02 03 04)

Click **Send** button

Example - Standard OBD-II request:

ID: `7DF`

Extended ID: ☐

FD: ☐

Data: `02 01 0C` (request engine RPM)

Example - CAN FD diagnostic:

ID: `744`

Extended ID: ☐

FD: ☑

BRS: ☑

Data: `02 10 01` → Click **Pad 00** → auto-fills to 64 bytes

__________________________________________________

## 10.3 How to Filter Messages

Goal: Show only specific CAN IDs

Scenario: You only want to see diagnostic responses (0x74C)

Steps:

Go to **Filters** tab

Enter filter details:

- Filter Name: Diagnostic Response

- Filter Type: Single

- Single ID: 74C

- Accept/Reject: Accept (show matching)

Click **Add Filter**

Ensure **Enabled** checkbox is checked

Return to **Main** tab - only matching messages will appear

Scenario: Hide all messages in range 0x100-0x1FF

Steps:

Go to **Filters** tab

Enter filter details:

- Filter Name: Hide low IDs

- Filter Type: Range

- From: 100

- To: 1FF

- Accept/Reject: Reject (hide matching)

Click **Add Filter**

__________________________________________________

## 10.4 How to Set Up Periodic Messages

Goal: Send a message automatically every X milliseconds

Scenario: Send "Tester Present" every 2 seconds to keep session alive

Steps:

Go to **Periodic** tab

Enter message details:

- ID: 7DF

- Data: 02 3E 00

- Interval (ms): 2000 (2 seconds)

- Count: 0 (infinite, or enter number for limited sends)

Click **Add**

Click **Start All** to begin transmission

Watch the Main tab - message appears every 2 seconds

Click **Stop All** when finished

__________________________________________________

## 10.5 How to Use Message History

Goal: Resend a message you sent earlier

Steps:

Send some messages normally

Go to **History** tab

You'll see all previously sent messages with timestamps

To resend exact same message:

- Select the message

- Click Resend

To modify before sending:

- Select the message

- Click Load (copies to Send panel)

- Modify as needed

- Click Send

__________________________________________________

## 10.6 How to Use Predefined Messages

Goal: Quickly send frequently used diagnostic messages

Steps to send predefined:

Go to **Predefined** tab

Select a message (e.g., "OBD Engine RPM")

Click **Send**

Steps to add your own predefined:

In **Send Panel**, set up your message (ID, data, flags)

Go to **Predefined** tab

Click **Add Current**

Your message is now saved for quick access

__________________________________________________

## 10.7 How to Monitor Specific ECU Communication

Goal: Analyze communication with a specific ECU (e.g., diagnostic tool ↔ ECU)

Scenario: Monitor diagnostic session with ECU using IDs 0x744 (request) and 0x74C (response)

Steps:

Connect to CAN bus

Go to **Filters** tab

Add filter for requests:

- Name: Diag Request

- Type: Single

- ID: 744

- Accept: ☑

Add filter for responses:

- Name: Diag Response

- Type: Single

- ID: 74C

- Accept: ☑

Go to **Main** tab

Click **▶ Start** to begin receiving

Enable **Show ASCII** to see text in data

Click **Edit Comments** and add:

- ID 744 → Comment: ECU Request

- ID 74C → Comment: ECU Response

Now you see only diagnostic traffic with clear labels

__________________________________________________

## 10.8 How to Export Communication Log

Goal: Save all messages to a file for later analysis

Steps:

Capture the messages you want (receive/send)

In the toolbar, click **Export TXT**

Choose location and filename

Click **Save**

The file contains:

CanOEs - Message Log

Exported: 2025-12-05 14:30:00

=====================================

Time          Dir  ID       DLC  Data                      ASCII     Flags  Comment

12:00:01.123  TX   0x744    3    02 10 01                  ...              Diag Request

12:00:01.456  RX   0x74C    8    06 50 01 00 19 01 F4 00   .P......         Diag Response

__________________________________________________

## 10.9 How to Analyze Message Patterns (Grouped View)

Goal: See how often each CAN ID appears and its latest data

Steps:

Start receiving messages

Go to **Grouped** tab

Click **Refresh** to see statistics

Table shows:

- How many times each ID was received

- Last data payload for each ID

- Last timestamp

Use this to identify:

- Most active IDs on the bus

- Which IDs have changing vs static data

- Missing expected IDs

__________________________________________________

## 10.10 How to Switch Themes

Goal: Change between light and dark mode

Steps:

Click **Toggle Theme** button (top-left corner)

Interface switches between:

- Light: White background (default, good for bright environments)

- Dark: Dark background (reduces eye strain in dim lighting)

__________________________________________________

# 11. Troubleshooting

## Connection Issues

| Problem | Solution |
| --- | --- |
| "Cannot load vxlapi64.dll" | Install Vector driver package |
| "Channel not found" | Check VN1640A is connected and powered |
| "No license" | Ensure valid Vector license is installed |
| "Access denied" | Close other applications using the channel |

## Communication Issues

| Problem | Solution |
| --- | --- |
| No messages received | Check bus termination (120Ω resistors) |
| TX messages fail | Verify baudrate matches other devices |
| CAN FD not working | Ensure all devices support CAN FD |
| Garbled data | Check baudrate settings on all nodes |

## Application Issues

| Problem | Solution |
| --- | --- |
| GUI freezes | Stop receiving, reduce message rate |
| High memory usage | Clear message list periodically |
| Theme not applying | Restart application |

__________________________________________________

# Appendix A: Keyboard Shortcuts

| Shortcut | Action |
| --- | --- |
| `Enter` (in Send panel) | Send message |
| `Ctrl+C` | Copy selected messages |
| `Delete` | Remove selected item (in lists) |

__________________________________________________

# Appendix B: CAN FD Data Lengths

Valid CAN FD payload lengths:

| DLC | Bytes |
| --- | --- |
| 0-8 | 0-8 |
| 9 | 12 |
| 10 | 16 |
| 11 | 20 |
| 12 | 24 |
| 13 | 32 |
| 14 | 48 |
| 15 | 64 |

__________________________________________________

# Appendix C: Common Diagnostic IDs

| ID | Description |
| --- | --- |
| 0x7DF | OBD-II functional broadcast address |
| 0x7E0 | OBD-II ECU physical request |
| 0x7E8 | OBD-II ECU physical response |
| 0x700-0x7FF | Standard diagnostic range |
| 0x744 | Common diagnostic request (vehicle-specific) |
| 0x74C | Common diagnostic response (vehicle-specific) |

__________________________________________________

© 2025 CanOEs Project